TECHNICAL PACK

# DELTONIANS

Please read in full and contact us in advance with any questions.	Current as of: [DATE]

| Members 5 | IEM Sends Required 5 × balanced XLR | Channels to FOH Up to 10 | Monitor Alternative Wedges accepted |
| --- | --- | --- | --- |

Overview

We are self-contained by default. We carry our own mixer, IEMs, and all sources listed below. We can operate independently of house infrastructure.

House PA / FOH engineer: Happy to work with venue sound tech. All outputs to FOH are balanced XLR unless stated.

House backline / drum kit: We are happy to use house backline and drum kit. Drummer will bring breakables. See individual sections for what changes when using venue-provided kit.

Monitors: Default is IEM. We carry our own in-ear units (XLR input). If you prefer wedges, please advise in advance.

Stage Plot

| ↑ UPSTAGE / BACK OF STAGE ↑ |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | [DRUMS] Back · Centre |  |  |
| [GUITAR] Front Far Left* | [VIOLIN] Front Left | [VOCALS] Front Centre | [BASS] Front Right |  |
| ↓ DOWNSTAGE / AUDIENCE ↓ * Guitar and bass may alternatively position either side of the drum kit depending on stage size. Confirm on the day. • All positions as seen from the audience. |  |  |  |  |

Channel List — Self-Contained WITH Electronic Drums

All outputs are at line level unless noted. All XLR outputs are balanced.

| Ch | Source | Signal / Connector | 48v | Notes |
| --- | --- | --- | --- | --- |
| 1 | Kick | Unbalanced 6.35mm jack or balanced XLR | - | Electronic drums — line level out. |
| 2 | Snare | Unbalanced 6.35mm jack or balanced XLR | - | Electronic drums — line level out |
| 3 | Toms | Unbalanced 6.35mm jack or balanced XLR | - | All toms on a single summed mono channel. |
| 4 | Cymbals | Unbalanced 6.35mm jack or balanced XLR | - | Hi-hats, ride and crash on one channel. |
| 5 | Click | Unbalanced 6.35mm jack or balanced XLR | No | Only if using IEMs. Route to IEMs only, not FoH |
| 6 | Cowbell | Balanced XLR | No | Only used If using house kit with no cowbell |
| 7 | Bass (DI) | Balanced XLR | No | Line 6 Pod Go. Requires 1× mains socket. Direct to FOH — no cab mic needed unless using house backline. |
| 8 | Electric Violin | Balanced XLR | No | Line 6 Helix HX Effects. |
| 9 | Guitar | Balanced XLR | No | Line 6 Helix HX Effects. |
| 10 | Vocals | Balanced XLR | No | We supply mic + effects unit. |

If using venue backline, sound tech to adjust channels to suit.

In-Ear Monitors & Stage Monitoring

Default: IEM. Each member carries their own in-ear unit accepting a balanced XLR input. Please provide one XLR send per person from your monitor desk (or our own desk).

Wedge alternative: Happy to use wedges if preferred — please advise in advance. One wedge per person minimum.

Click if being used: Must appear in ALL IEM mixes. It must NOT appear in wedges or FOH.

Starting mix: Each band member wants a full mix with themselves slightly higher than everyone else. We will adjust from there in sound check.

Stage positions and Power Requirements

| Item | Position | Sockets | Notes |
| --- | --- | --- | --- |
| Drums | Back centre | 2 × | Drum module Laptop |
| Bass | Front right / rear right | 1 × | Line 6 Pod Go |
| Guitar | Front far left / rear left | 1 × | Line 6 Helix |
| Violin | Front left | 1 × | Line 6 Helix |
| Vocals | Front centre | 1 × | Vocal effects unit |
| TOTAL (minimum) | — | 6 × | All standard 13A. Ideally one clean power strip per side of stage. |

General Notes

Self-contained setup and soundcheck: When using our own equipment, setup from scratch usually takes 1h30m from arrival to start of sound check. Depending on venue, sound check takes around 20-25 minutes.

Venue setup and soundcheck – situation dependent.

Contact: Al Provan, DM us on Facebook or Instagram